Attachment D

Computation Sheet for Determining Runoff Coefficients

INSTRUCTIONS

- The runoff coefficient (“C” value) is used to estimate the impact on stormwater runoff due to development of a site.  The C value is the amount of rainfall that becomes runoff.  The less runoff that is absorbed into the ground, the higher the C value.

- Refer to the local Hydrology Manual for requirements for calculating weighted runoff coefficients for areas containing varying amounts of different cover.

- Refer to the local Hydrology Manual for information of Runoff Coefficients for Undeveloped Areas and Runoff Coefficients for Developed Areas.

EXAMPLE

Total Site Area					=	120 Acres	(A)

Existing Site Conditions

Impervious Area1					=	20 Acres	(B)

Impervious Area Runoff Coefficient 2		=	0.90		(C)

20 x 0.90		=	18 Acres 	(B x C)

Pervious Area3					=	102 Acres	(D)

Pervious Area Runoff Coefficient			=	0.4		(E)

104 x 0.4		=	41.6 Acres	(D x E)

Sum:	18 + 41.6		=	59.6	(B x C) + (D x E)

Divide: 59.6/120		=	0.50

Existing Area Runoff Coefficient			=	0.50		(F)

Proposed Site Conditions

Impervious Area1					=	100 Acres	(G)

Impervious Area Runoff Coefficient2			=	0.95		(H)

100 x 0.95		=	95 Acres 	(G x H)

Pervious Area						=	20 Acres	(I)

Pervious Area Runoff Coefficient			=	0.4		(J)

20 x 0.4			=	8 Acres 	(I x J)

Sum:	95 + 8			=	103	(G x H) + (I x J)

Divide: 103/120		=	0.86

Existing Area Runoff Coefficient			=	0.86		(F)

REQUIRED TEXT:

| Total Site Area | = | Acres |  | (A) |
| --- | --- | --- | --- | --- |

## Existing Site Conditions

| Impervious Site Area 1 |  | = | Acres |  | (B) |
| --- | --- | --- | --- | --- | --- |
| Impervious Site Area Runoff Coefficient 2, 4 |  | = |  |  | (C) |
| Pervious Site Area 3 |  | = | Acres |  | (D) |
| Pervious Site Area Runoff Coefficient 4 |  | = |  |  | (E) |
| Existing Site Area Runoff Coefficient | B×C+D×E(A) | = |  |  | (F) |

## Proposed Site Conditions (after construction)

| Impervious Site Area 1 |  |  | = |  | Acres |  |  |  | (G) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Impervious Site Area Runoff Coefficient 2, 4 |  |  | = |  |  |  |  |  | (H) |
| Pervious Site Area 3 |  |  | = |  | Acres |  |  |  | (I) |
| Pervious Site Area Runoff Coefficient 4 |  |  | = |  |  |  |  |  | (J) |
| Proposed Site Area Runoff Coefficient | G×H+I×J(A) | = |  |  |  |  |  | (K) |  |

- Includes paved areas, areas covered by buildings, and other impervious surfaces.

- Use 0.95 unless lower or higher runoff coefficient can be verified.

- Includes areas of vegetation, most unpaved or uncovered soil surfaces, and other pervious areas.

- Refer to local Hydrology Manual for typical C values.