Attachment S

Pollutant Testing Guidance Table

INSTRUCTIONS

- The following Table will be updated periodically as more information becomes available.

| Attachment S Pollutant Testing Guidance Table 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Category | Construction Site Material | Visually Observable? | Pollutant Indicators 2 | Suggested Analyses Field 3 | Laboratory |
| Asphalt Products | Hot Asphalt | Yes - Rainbow Surface or Brown Suspension | Visually Observable - No Testing Required |  |  |
|  | Asphalt Emulsion |  |  |  |  |
|  | Liquid Asphalt (tack coat) |  |  |  |  |
|  | Cold Mix |  |  |  |  |
|  | Crumb Rubber | Yes – Black, solid material | Visually Observable - No Testing Required |  |  |
|  | Asphalt Concrete (Any Type) | Yes - Rainbow Surface or Brown Suspension | Visually Observable - No Testing Required |  |  |
| Cleaning Products | Acids | No | pH Acidity Anions (acetic acid, phosphoric acid, sulfuric acid, nitric acid, hydrogen chloride) | pH Meter Acidity Test Kit | EPA 150.1 (pH) |
|  |  |  |  |  | SM 2310B (Acidity) |
|  |  |  |  |  | EPA 300.0 (Anion) |
|  | Bleaches | No | Residual Chlorine | Chlorine | SM 4500-CL G (Res. Chlorine) |
|  | Detergents | Yes - Foam | Visually Observable - No Testing Required |  |  |
|  | TSP | No | Phosphate | Phosphate | EPA 365.3 (Phosphate) |
|  | Solvents | No | VOC | None | EPA 601/602 or EPA 624 (VOC) |
|  |  |  | SVOC | None | EPA 625 (SVOC) |
| Portland Concrete Cement & Masonry Products | Portland Cement (PCC) | Yes - Milky Liquid | Visually Observable - No Testing Required |  |  |
|  | Masonry products | No | pH | pH Meter Alkalinity or Acidity Test Kit | EPA 150.1 (pH) |
|  |  |  | Alkalinity |  | SM 2320 (Alkalinity) |
|  | Sealant (Methyl Methacrylate - MMA) | No | Methyl Methacrylate | None | EPA 625 (SVOC) |
|  |  |  | Cobalt |  | EPA 200.8 (Metal) |
|  |  |  | Zinc |  |  |
|  | Incinerator Bottom Ash Bottom Ash Steel Slag Foundry Sand Fly Ash Municipal Solid Waste | No | Aluminum Calcium Vanadium Zinc | Calcium Test | EPA 200.8 (Metal) EPA 200.7 (Calcium) |
|  | Mortar | Yes - Milky Liquid | Visually Observable - No Testing Required |  |  |
|  | Concrete Rinse Water | Yes - Milky Liquid | Visually Observable - No Testing Required |  |  |
|  | Non-Pigmented Curing Compounds | No | Acidity | pH Meter Alkalinity or Acidity Test Kit | SM 2310B (Acidity) |
|  |  |  | Alkalinity |  | SM 2320 (Alkalinity) |
|  |  |  | pH |  | EPA 150.1 (pH) |
|  |  |  | VOC |  | EPA 601/602 or EPA 624 (VOC) |
|  |  |  | SVOC |  | EPA 625 (SVOC) |
| Landscaping and Other Products | Aluminum Sulfate | No | Aluminum | TDS Meter Sulfate | EPA 200.8 (Metal) |
|  |  |  | TDS |  | EPA 160.1 (TDS) |
|  |  |  | Sulfate |  | EPA 300.0 (Sulfate) |
|  | Sulfur-Elemental | No | Sulfate | Sulfate | EPA 300.0 (Sulfate) |
|  | Fertilizers-Inorganic 4 | No | Nitrate | Nitrate | EPA 300.0 (Nitrate) |
|  |  |  | Phosphate | Phosphate | EPA 365.3 (Phosphate) |
|  |  |  | Organic Nitrogen | None | EPA 351.3 (TKN) |
|  |  |  | Potassium | None | EPA 200.8 (Metal) |
|  | Fertilizers-Organic | No | TOC | Nitrate | EPA 415.1 (TOC) |
|  |  |  | Nitrate |  | EPA 300.0 (Nitrate) |
|  |  |  | Organic Nitrogen |  | EPA 351.3 (TKN) |
|  |  |  | COD |  | EPA 410.4 (COD) |
|  | Natural Earth (Sand, Gravel, and Topsoil) | Yes - Cloudiness and turbidity | Visually Observable - No Testing Required |  |  |
|  | Herbicide | No | Herbicide | None | Check lab for specific herbicide or pesticide |
|  | Pesticide |  | Pesticide |  |  |
|  | Lime |  | Alkalinity | pH Meter Alkalinity or Acidity Test Kit | SM 2320 (Alkalinity) |
|  |  |  | pH |  | EPA 150.1 (pH) |
| Painting Products | Paint | Yes | Visually Observable - No Testing Required |  |  |
|  | Paint Strippers | No | VOC | None | EPA 601/602 or EPA 624 (VOC) |
|  |  |  | SVOC | None | EPA 625 (SVOC) |
|  | Resins | No | COD | None | EPA 410.4 (COD) |
|  |  |  | SVOC |  | EPA 625 (SVOC) |
|  | Sealants | No | COD | None | EPA 410.4 (COD) |
|  | Solvents | No | COD | None | EPA 410.4 (COD) |
|  |  |  | VOC |  | EPA 601/602 or EPA 624 (VOC) |
|  |  |  | SVOC |  | EPA 625 (SVOC) |
|  | Lacquers, Varnish, Enamels, and Turpentine | No | COD | None | EPA 410.4 (COD) |
|  |  |  | VOC |  | EPA 601/602 or EPA 624 (VOC) |
|  |  |  | SVOC |  | EPA 625 (SVOC) |
|  | Thinners | No | VOC | None | EPA 601/602 or EPA 624 (VOC) |
|  |  |  | COD |  | EPA 410.4 (COD) |
| Portable Toilet Waste Products | Portable Toilet Waste | Yes | Visually Observable - No Testing Required |  |  |
| Contaminated Soil 5 | Aerially Deposited Lead3 | No | Lead | None | EPA 200.8 (Metal) |
|  | Petroleum | Yes – Rainbow Surface Sheen and Odor | Visually Observable - No Testing Required |  |  |
|  | Other | No | Contaminant Specific | Contaminant Specific | Contaminant Specific |
| Line Flushing Products | Chlorinated Water | No | Total chlorine | Chlorine | SM 4500-CL G (Res. Chlorine) |
| Adhesives | Adhesives | No | COD | None | EPA 410.4 (COD) |
|  |  |  | Phenols | Phenol | EPA 420.1 (Phenol) |
|  |  |  | SVOC | None | EPA 625 (SVOC) |
| Dust Palliative Products | Salts (Magnesium Chloride, Calcium Chloride, and Natural Brines) | No | Chloride | Chloride | EPA 300.0 (Chloride) |
|  |  |  | TDS | TDS Meter | EPA 160.1 (TDS) |
|  |  |  | Cations (Sodium, Magnesium, Calcium) | None | EPA 200.7 (Cations) |
| Vehicle | Antifreeze and Other Vehicle Fluids | Yes - Colored Liquid | Visually Observable - No Testing Required |  |  |
|  | Batteries | No | Sulfuric Acid | None | EPA 300.0 (Sulfate) |
|  |  |  | Lead | None | EPA 200.8 (Metal) |
|  |  |  | pH | pH Meter Alkalinity or Acidity Test Kit | EPA 150.1 (pH) |
|  | Fuels, Oils, Lubricants | Yes - Rainbow Surface Sheen and Odor | Visually Observable - No Testing Required |  |  |
| Soil Amendment/Stabilization Products | Polymer/Copolymer 6, 7 | No | Organic Nitrogen | None | EPA 351.3 (TKN) |
|  |  |  | BOD | None | EPA 405.1 (BOD) |
|  |  |  | COD | None | EPA 410.4 (COD) |
|  |  |  | DOC | None | EPA 415.1 (DOC) |
|  |  |  | Nitrate | Nitrate | EPA 300.0 (Nitrate) |
|  |  |  | Sulfate | Sulfate | EPA 300.0 (Sulfate) |
|  |  |  | Nickel | None | EPA 200.8 (Metal) |
|  | Straw/Mulch | Yes - Solids | Visually Observable - No Testing Required |  |  |
|  | Lignin Sulfonate | No | Alkalinity | Alkalinity | SM 2320 (Alkalinity) |
|  |  |  | TDS | TDS Meter | EPA 160.1 (TDS) |
|  | Psyllium | No | COD | None | EPA 410.4 (COD) |
|  |  |  | TOC |  | EPA 415.1 (TOC) |
|  | Guar/Plant Gums | No | COD | None | EPA 410.4 (COD) |
|  |  |  | TOC |  | EPA 415.1 (TOC) |
|  |  |  | Nickel |  | EPA 200.8 (Metal) |
|  | Gypsum | No | pH | pH Meter Alkalinity or Acidity Test Kit | EPA 150.1 (pH) |
|  |  |  | Calcium | Calcium | EPA 200.7 (Calcium) |
|  |  |  | Sulfate | Sulfate | EPA 300.0 (Sulfate) |
|  |  |  | Aluminum | None | EPA 200.8 (Metal) |
|  |  |  | Barium |  |  |
|  |  |  | Manganese |  |  |
|  |  |  | Vanadium |  |  |
| Treated Wood Products | Ammoniacal-Copper-Zinc-Arsenate (ACZA) Copper-Chromium-Arsenic (CCA) Ammoniacal-Copper-Arsenate (ACA) Copper Naphthenate | No | Arsenic | Total Chromium | EPA 200.8 (Metal) |
|  |  |  | Total Chromium |  |  |
|  |  |  | Copper |  |  |
|  |  |  | Zinc |  |  |
|  | Creosote | Yes - Rainbow Surface or Brown Suspension | Visually Observable - No Testing Required |  |  |

Notes:

- 1 If specific pollutant is known, analyze only for that specific pollutant. See MSDS to verify.

- For each construction material, test for one of the pollutant indicators. Bolded pollutant indicates lowest analysis cost or best indicator. However, the composition of the specific construction material, if known, is the first criterion for selecting which analysis to use.

- See www.hach.com, www.lamotte.com, www.ysi.com and www.chemetrics.com for some of the test kits

- If the type of inorganic fertilizer is unknown, analyze for all pollutant indicators listed.

- Only if special handling requirements are required in the contract documents for aerially deposited lead (ADL)

- If used with a dye or fiber matrix, it is considered visually observable and no testing is required.

- Based upon research conducted by the State of California Department of Transportation (Caltrans), the following copolymers/polymers do not discharge pollutants and water quality sampling and analysis is not required: Super Tak™, M-Binder™, Fish Stik™, Pro40dc™, Fisch-Bond™, and Soil Master WR™.